The interaction of silver ions and hydrogen peroxide in the inactivation of E. coli: a preliminary review of a new long-acting residual drinking water disinfectant

Water Science and Technology Vol 31 No 5-6 pp 123–129 © IWA Publ 1995

Rami Pedahzur, Ovadia Lev, Badri Fattal and Hillel I. Shuval

Division of Environmental Sciences, The Fredy and Nadine Herrmann School of Applied Science, The Hebrew University of Jerusalem Jerusalem 91904 Israel

ABSTRACT
The inactivation efficiencies of silver ions, hydrogen peroxide and their combination was studied as part of a performance review of the combined disinfectant for drinking water applications. Both, silver and hydrogen peroxide exhibited a significant inactivation performance even at concentrations that do not pose any health risk according to the EEC, WHO and the USEPA (the USEPA Maximum Contaminant Level (MCL) of silver is 90 ppb, and currently, there is no MCL for hydrogen peroxide but it is approved as a food additive in the USA). Combinations of 1:1000 silver:hydrogen peroxide (w) exhibited higher inactivation performance as compared with each of the disinfectants alone and in some cases a synergistic effect was observed, i.e., the combined disinfectant exhibited higher inactivation performance than the sum of the inactivation levels of the separate disinfectants. Thus, for example, one hour exposure to 30 ppb silver, 30 ppm hydrogen peroxide and their combination yielded 2.87, 0.65 and 5 logs of inactivation respectively.

…………………………………………………………………………………………………………

Water Science and Technology Vol 35 No 11-12 pp 87–93 © IWA Publ 1997

Silver and hydrogen peroxide as potential drinking water disinfectants: their bactericidal effects and possible modes of action

R. Pedahzur*, H. I. Shuval* and S. Ulitzur**

*Department of Environmental Sciences, The Hebrew University of Jerusalem, Jerusalem, Israel
**Department of Food Engineering and Biotechnology, Technicon Haifa 32000 Israel

ABSTRACT
Silver and hydrogen peroxide (HP) acted synergistically on the viability of E. coli K-12. In certain concentration ranges the synergistic effect amounted to about 3 logs. Toxicity process kinetics were determined by following the decrease in luminescence of a highly luminescent recombinant E. coli harbouring a plasmid carrying the whole lux system of Vibrio fischeri. As in the viability studies, silver and HP also showed a synergistic effect on in vivo luminescence, which amounted to a 2 log increase in toxicity. A similar phenomenon was found for silver and certain metal ions, including Cu2+, Ni2+, Zn2+ and Cd2+, where toxicity increased by a factor of 10 in the presence of sub-inhibitory concentrations of silver. It appears that the combined toxic effect of silver and HP may be related with damage to cellular proteins. Nevertheless, the involvement of other cellular moieties cannot be ruled out. The possibility that the synergistic effect is related to chemical interactions between silver and HP and the consequent increase in their toxicity is discussed.

………………………………………………………………………………………………………

Water Science and Technology Vol 42 No 1-2 pp 293–298 © IWA Publishing 00

The efficacy of long-lasting residual drinking water disinfectants based on hydrogen peroxide and silver

R Pedahzur*, D Katzenelson*, N Barnea*, O Lev* HI Shuval*, B Fattal* and S Ulitzur**

* Division of Environmental Sciences, the Freddy and Nadine Herrmann School of Applied Science, Hebrew University of Jerusalem, Jerusalem, Israel 
** Department of Food Engineering and Biotechnology, Technicon-Israel Institute of Technology, Haifa 32000, Israel

ABSTRACT
The aim of the present work was to review the disinfectant capacity and the possible fields of application of a combined silver and hydrogen peroxide (HP) water disinfectant. The findings demonstrated the high bactericidal action of silver (on E. coli) and its relatively ineffective virucidal effect (on MS-2 phage). HP was found to have a small bactericidal effect and a mild virucidal one. When combined, silver and HP usually exhibited a synergistic action on the viability of E. coli and on the luminescence of recombinant luminescent E. coli. In some instances, the combined bactericidal effects were 1000-fold higher than the sum of the separate ones. No increased virucidal action was observed. The biocidal action of the combination generally increased with increasing temperature and pH. The physiological effects and mechanisms of toxicity of HP, silver and their combinations, were assessed by monitoring the induction of stress promoters upon exposure to the active agents, and by assessing the sensitivity of E. coli mutated in major stress responses to HP, silver and their combinations. The results showed that HP induced a wide array of stress responses, that both silver and HP induced promoters regulated by the heat shock response, and that the dnaK promoter (regulated by the heat shock response) was synergistically induced. The mutant sensitivity tests showed that bacteria deficient in the ability to activate central cellular stress responses were hypersensitive to both HP and silver. These results imply that cellular proteins, and possibly the DNA, are the cellular moieties chiefly affected. The above findings suggest that the potentiated effect of HP and silver is a metabolically dependant/related process that stems from a combination and/or accumulation of physiological effects exerted by the active ingredients. The physico-chemical properties of the combined disinfectant, and its disinfection capacity, points to its potential application as a long-term secondary residual disinfectant for water of relatively high quality.